Pierce BCA Protein Assay Protocol

פרוטוקול מלא למדידה ב-ELISA reader

Date: ____________

Experiment: _____________________________________________________________

- Heat oven/water bath to 37°C (or can use PCR if using PCR 96 plate /strips)

- Dilute BSA Standards

- BSA stock: 2mg/ml (from ampules of kit, kept in aliquots -20 BSA box)

| Vial | BSA source (l) | DDW (l) | Final conc. (g/l) |
| --- | --- | --- | --- |
| 2 | Stock | 0 | 2 |
| 1 | 50 of stock | 50 | 1 |
| 0.5 | 50 of 1 | 50 | 0.5 |
| 0.25 | 50 of 0.5 | 50 | 0.25 |
| 0.1 | 40 of 0.25 | 60 | 0.1 |
| Blank | 0 | 50 | 0 |

- Prepare BCA working reagent (WR, 50+1 dilution of Reagent A+B).

| WR | X1 | X(20 + [___ samples X4])= _____ |
| --- | --- | --- |
| Reagent A | 200l |  |
| Reagent B | 4 l |  |
| Total | 200 l |  |

- 20 = 6 standards in triplicates + extra to allow for sample loss +2 for lysis buffer blank

- Number of samples X 4 = each sample run in duplicates of two concentrations

- WR is stable for up to 1 week at RT.

- Prepare a 1:5 dilution of samples: 6µl sample + 24µl Lysis buffer (for duplicates).

Samples:

- Distribute 10l triplicates of each standard, and 10µl of diluted and undiluted sample in duplicates, in wells of 96w flat-buttom plate/96 well PCR plate (covered).

- Add 200l WR to each well, cover, and mix on plate shaker 850 rpm for 30".

- Incubate at 37°C for 30' (can increase to 2 hrs). Cool to RT (can be in water bath or PCR or oven).

- Measure at 562 (540-590) nm on ELISA reader after 5' or more (less than an hour).

- Don't forget to take USB to get measurement results.

- Substract average Blank readings, and Plot standard curve (quadratic or best-fit).

- Samples:  Substract lysis buffer read from all values

Pierce BCA Protein Assay Protocol

פרוטוקול מותאם לכמות קטנה ומדידה ידנית בננוטדרופ

Date: ____________

Experiment: _____________________________________________________________

- Heat oven/water bath to 37°C (or can use PCR if using PCR 96 plate /strips)

- Dilute BSA Standards

- BSA stock: 2mg/ml (from ampules of kit, kept in aliquots -20 BSA box)

| Vial | BSA source (l) | DDW (l) | Final conc. (g/l) |
| --- | --- | --- | --- |
| 2 | Stock |  | 2 |
| 1 | 10 of stock | 10 | 1 |
| 0.5 | 10 of 1 | 10 | 0.5 |
| 0.25 | 10 of 0.5 | 10 | 0.25 |
| 0.1 | 4 of 0.25 | 6 | 0.1 |
| Blank | 0 | 10 | 0 |

- Prepare BCA working reagent (WR, 50+1 dilution of Reagent A+B).

| WR | X1 | X(20 + [___ samples X4])= _____ |
| --- | --- | --- |
| Reagent A | 40 l |  |
| Reagent B | 0.8 l |  |
| Total | 40 l |  |

- 20 = 6 standards in triplicates + extra to allow for sample loss +2 for lysis buffer blank

- Number of samples X 4 = each sample run in duplicates of two concentrations

- WR is stable for up to 1 week at RT.

- Prepare a 1:5 dilution of samples: 2µl sample + 8µl Lysis buffer (for duplicates).

Samples:

- Distribute 2l triplicates of each standard, and 2µl of diluted and undiluted sample in duplicates, in wells of 96w flat-buttom plate/96 well PCR plate (covered).

- Add 40l WR to each well, cover, and mix on plate shaker 850 rpm for 30".

- Incubate at 37°C for 30' (can increase to 2 hrs). Cool to RT (can be in water bath or PCR or oven).

- Measure after 5' or more (less than an hour) on nanodrop colorimetric app/BCA assay function (Blank with water)

- Don't forget to take USB to get measurement results.

- Samples:  Substract lysis buffer read from all values

Date: ____________

Experiment: _____________________________________________________________

| Sample | 1:5 | 1:5 | Ave-lysis buf | undiluted | undiluted | Ave-lysis buf | Final µg/µl |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Lysis buffer |  |  |  |  |  |  |  |