Western Blotting – Invitrogen system

Date: _______________

Project: ____________________________________________________________________

Gel type (%): ___________________________________________; Homemade/Precast

Samples preparation

- Prepare sample buffer and appropriate reducing agent

- If using Laemmli sample buffer (LSB) + β-mercaptoETOH (Keep 4ºC for 1-2 months)

  - 50µl of β-ME per 950µl LSBx2; 100µl of β-ME per 900µl LSBx4; 125µl of β-ME per 875µl LSBx5.

- Note: β-mercaptoETOH is a severe irritant and is readily absorbed through the skin; use in a fume hood.

- Prepare samples:

- Dilute samples with PBS X1 for equal loading volumes, as necessary.

- Typical loading amount is 30-40µg.

- Dilute 1:1 sample to LSBx2+β, 3:1 for LSBx4+β, 4:1 for LSBx5+β.

| Sample | Conc. (µg/µl) | Sample (µl, for ____ µg) | + PBS (µl) | LSB X__ buffer (µl) | Total (µl) |
| --- | --- | --- | --- | --- | --- |
| Size standard | - | - | - | - | 5 |

- Boil samples 5' or keep 30' at 37ºC, put cap holder on tubes before boiling

- Do not heat standard, Keep on ice

Running samples on homemade/precast gel (Invitrogen Mini Gel Tank)

- Make 400 ml Running buffer X1 for each Mini Gel Tank chamber to be used.

- Assemble Mini Gel Tank chamber and fill with Running buffer X1 up to cathode.

- Rinse wells 3 times w/ buffer and place cassette into chamber with wells facing forward. Hold the cassette in a raised position and close clamp.

  - If using precast gel, peel away the tape at the bottom before inserting the gel cassette

  - If running only one gel, remove the second cassette clamp

- Optional: load small amount of sample buffer into each well and run for a minute to make sure all wells are in order.

- Put in well guide to assist loading and carefully load samples and size standard into gel:

1.______ 2.______ 3.______ 4.______ 5.______ 6.______ 7.______ 8.______ 9.______ 10.______

- Can wash tip in running buffer for reuse between samples

- Load up to 40 µl DDW carefully in each well on top of samples

- Open clamp and slowely lower the gel cassette

- Run gel until the blue line disappears:

- For homemade gels: run at 130-200V (approx 1h)

- For precast gels: run at 225V (25-40') or refer to specific instructions

- Carefully remove gel (using the well guide or spatula) and place in Transfer buffer X1 or DDW (if using "Transfer stacks") for equilibration 5-30'. Cut a corner of gel to mark orientation

- Start time: ___________; _______V, _______mA

Finish time: ___________; _______V, _______mA

Membrane transfer – semi-dry (Invitrogen Power Blotter)

  - Perform blotting within 15' of sandwich assmebly

  - Make sure to remove all bubbles during assembly

  - Handle membrane carefully with clean gloves

- If using "Transfer Stacks":

  - No need for Transfer buffer

  - Carefully place bottom stack on the anode

  - Cut a corner of the membrane to mark orientation and place the gel on the membrane

  - Place top stack w/o the separator

  - Lock casette with cathode and slide into the Blotter

- If making your own stack or using pre-cut stacks:

  - Make 50 ml Transfer buffer X1

- Cut 4 Whatman papers and 1 membrane to size

- Cut a corner of the membrane to mark orientation

- Equilibrate papers + membrane in Transfer buffer X1 5'

- Asseble on anode (buttom): 
Whatman x2  membrane  gel  Whatman x2

- Lock casette with cathode and slide into the Blotter

- Choose correct program in Power Blotter

- Choose between 1 or 2 mini gels to be transferred

- Choose transfer method by required protein sizes

- Pre-programmed transfer methods:

- For large proteins >150 kDa, may need to increase the run time to 8-10 min

- For small proteins <25 kDa, may need to reduce the run time to 4-6 min

- Post-transfer:

- Stain membrane with Ponceau (save for reuse); mark lanes and marker values.

- Wash ponceau off membrane by shaking in PBS X1.

- Keep membrane refrigerated, wrapped in saran wrap and wet with PBS X1.

Optional: Coomassie Blue gel staining

- Stain gel with 0.1% Coomassie Blue solution up to 1 hr.

- Destain gel in destaining solution (225 ml ETOH + 225 ml DDW + 50 ml glacial acetic acid), may take up to overnight RT on shaker.

- Use sponge cuts during destain.

Date: _______________

Project: ____________________________________________________________________

Blocking

- Make 30-50ml 5-10% dry milk in Tween20 0.2% and PBS X1 (PBS-T-M).

- Block membrane by incubating in PBS-T-M overnight 4ºC or 1-2 hr RT, on rocker.

- Rinse the membrane for 5" in PBS-T after blocking.

Primary antibody

- Incubate w/ primary ab diluted in 1-10 ml PBS-T-M or Advanblock 1-2 hr RT or overnight 4ºC, on rocker.

- Primary ab used: _____________________________________________________

- Dilution: ___________________________________________________________

- Wash in PBS-T (0.2%) 6x5' or 3x10' (make up to 1L).

- Keep primary ab for reuse

  - If diluted in PBS-T-M, add NaAzid for preservation

Secondary antibody

- Incubate w/ secondary ab diluted in 10 ml PBS-T-M on rocker for 1hr RT.

- Secondary ab used: ____________________________________________________

- Dilution: ____________________________________________________________

- Wash in PBS-T (0.2%) 6x5' or 3x10'.

- Wash in PBS-T (3%) 3' and in PBS X1 (w/o Tween) 10'.

- Keep sample of ab aside as markers for ECL.

ECL

- Mix both solutions in 1:1 ratio (~1 ml each solution per membrane).

- Make control HRP-ab (secondary ab) markers dotted on small membrane piece.

- Incubate membranes 1-3' on parafilm in dark box.

- Drip on blotting paper to remove excess ECL solution, shake slightly and expose membrane and control piece on Saran wrap.

- After filming, shake with PBS X1 for 10'. Wrap in saran wrap and keep refrigerated.

Film at TAMI RUBINEK's lab (Fusion Pulse TS VILBER LOURMAT)

- Make sure that the camera is set on 1 (upper right wheel)

- Press Chemiluminesence and put blot on tray 1

- Press Preview to adjust blot in the right place and adjust lighting

- Press Multiple jul and Auto (to adjust blot time exposure press Manual)

- Press start

- Mark the marker and press paste selection and OK

- Save all images in compatibility TIF or JPG

- Leave the software and camera on

Stripping (up to 2-3 times)

- Option 1: Mix 1.25 ml Tris (1M, pH=6.7) + 140 µl β-mercaptoETOH + 2 ml SDS 20%. Adjust volume to 20 ml with DDW. Soak at 50ºC for 30'. Wash with PBS-T 0.5% 2x10'.

- Option 2: Mix 2 ml Glycine (1M pH=2.5) in 20 ml DDW (1:10) per membrane. Soak for 30-60' RT a rocker. Wash 3 times in PBS before blocking.

- Option 3: Use commercial Stripping solution.

- React with ECL, make long exposure and film to ensure full stripping.

- Shake with PBS X1 for 10'. Wrap in saran wrap and keep refrigerated.